Hello, today my seminar will be talking about the comparison and implementation of data compression methods for soc based radar systems. To get a better understanding of what that actually means, lets look at the problem my thesis attemps to solve. With the rapid expansion of radar sensors in automotive systems, such as in cruise control and blind spot detection, it creates signficannt challenges inn managing the large amount of data coming from this radars. This increase in data requires a more expensive architecture with more expensive cabling and computing power needed, alongside a high latency between the radar being sent to being decoded.

To get a better undnerstanding of what this actually looks like, here is a typical architecture used inside of a car. Here you can see a radar being used at various sections in a car, with the front section have two radars and sides having one each. Each radar uses camera serial interface, or csi which has 4 parallell lines of data, with this example each having around 600Mbps of throughput. These parallel lines are then seerialised into one linne at around 2.5Gbps or the case of the two radars, 5 Gbps. This very high throughput is then sent to the deserialiser which is located at the system on chip, which then converts it back to the parallel lines. This architecture is called serialiser deseriliser or serdes, and has a couple of flaws. Mainly, the issue comes from the extremely high throughput from the serialiser being sent to the deserialiser, which requires really expensive shielded cabling alongside high computer power at the deserialiser.

An alternative approach to this problem is using compression. Essentially each radar module nnow has a compression block, using an MCU or more commonly an FPGA. The data is compressed down to a more accepablee level, around 1 Gbps or lower to allow the use of gigabit ethernent before being sent to the system on chip for decompression. My thesis will essentially be looking at various compression algorithms wee can use to ensure that data is compressed enough without losing too much information.

To get a better understanding of the surrounding compression algorithms work, lets have a look at how automotive radar works. Automotive radar works by sending a conntinnous pulse, commonly called frequennncy modulated continuous wave radar. Essentially, our transmit waveform from the radar looks like this, with a frequency modulated ramp conntinnously repeating, with each section being called a chirp. From the receive, we are able to deduce the distance, velocity and angle to the object the radar is seeing. Firstly, to get the distance to the object, we look at the phase shift between the transmit and receive chirp. By mixing these two signals, we can get the beat frequency or the intermediate frequency, which we then use the fft to get the time delay and thus the distance to the object. To get the velocity, this is where the frequency modulated part comes from. Since there would be no way to tell between phase shift and doppler shift from a moving target, we need a second variable to work with, which is a doppler across chirps. By doingn a second fft across chirps, we can calculate the doppler shift or the velocity of the object. Lastly, to get the angle the object is coming from, we use MIMO radars, or multiple input multiple output. By comparing the receive from each of the attenntenas we can nseee here, we cann see multiple objects by knowing where the object comes from. This gives us a 3d map you cann see here, which stores the distance velocity and angle to the object. The Texas instruments module we are using to get our radar data has 12 trannsmit and 16 receive antennas and so has a virtual antetnna array of 192. Sinnce the amount of data scales quadritcally with the addition of inndivudal transmit and receive antennas, the amount of data increases a lot when requirinnng more and more resolution.

Compressionn algorithms take advantage of these radar ffts to actually compress the data without too many or even no losses. To actually get a quanitifiable measurement of compression methods, there are a couple of key metrics that we can use. The first is the compression ratio, which is arguably the most important metric. Essentially it is the ratio of the input data to the output data. So if we have 32 input bits and 8 output bits, then our compression ratio is 4. Typically, a higher compression ratio means sacrifices in other metrics

Next we have false positives and false negatives. They are consequences of lossy compression, where we might think that an object is there inn a fasle positive, or think that the object isn’t there when it is, which is way worse

Lastly is the noise floor and the snr. These ensure that the compression algorithm isn’t full of noise, or remove any weaker signals

Lastly is latency, which the time it takes from the radar sending its data to decompression. Or the time taken to compress decompress.

Now that we have the technical background, we cann start looking at some of thee existing compression algorithms. Firslty, we have thee compression algorithms that are made by semiconductor manufacturers. We have Texas instruments, nxp and infinneon. NNotably, infinennon actually have their algorithms open source, whilst nxp do not, it is in the silicon of its microncotnroller digital signal processing unit. Next we have compression algorithms produced by academia. DRHE is a lossless algorithm which was specifically made for on the fly data compression (vs compression after receiving radar data). AdaRadar is a newer technique which uses neural networks for compression, achieving extremely high compression ratios with very little loss of data. Lastly, we have image compression techniques, such as the nnotably jpeg2000. BAQ is thee most used radar data compression technique, used specifically in nstatellites. All of these compression techniques listed are required to have very low latency, which is required by automotive purposes.

Now, after looking at some of the important compression technniques that currently exist in the market and in research, I wanted to look at a similar research paper that explored some of these compression algorithms. In their research, they did a simulation of the compression algorithms to get their metrics, andnn then implementnned the best algorithm onn ann fpga and compared to simulated results. Lookinng at their results, youcan nsee they compared the Texas insntrumentns and infineonn algorithm, but found that the drhe academic algorithm was the best since it had a good compression ratio with very little losses. In hardware, they did have some issues getting the same results inn simulation, but nevertheless gathered similar results.

So now that we’ve looked at some of the compression algorithms in the market and in academia, what are some of the gaps we can see. Firstly, we have fixed vs float quantisationn error, which we saw in the results of the masters thesis. As I mentionnned before, the DRHE algorithm that was implemented was meant to be lossless, but in hardware and simulation, thjey ended up having some losses due to quanisatipn error which Ill explain in a bit. Next is thee fact that the results gathered by the masters papeer did not explore the decompression after the compression, alongside the fact that they did not use real radar data, instead using simulated data to see the affects of false positives and nnegatives in real life. Lastly, is there are some compression algorithms that have yet to be compared against eeach other, like the nxp, baq, jpeegg and adaradar algorithms.

So yea, that is what my work will be on, exploring these gaps in research. Since this is the work I will be doing, I want to explore each of these points in greater detail. Firstly, we have the fixed vs float compression error. As I mentioned, DRHE is meant to be a lossless compression nalgorithm, however, there was still some eerrors. Looking at the baselie floating point arithmetic, we can nsee that there was a compression ratio of 1 annd no errors. However, when using fixed poinnnt arithmetic I the nenxt linne, we can nsee some losses. To get a better undnerstanding of why this occurs, when using fixed vs float, lets quickly understand what fixed and float is. Fixed point numbers have a fixed decimal point. This allows it to be simple in hardware to implem,ent. Float poinntn have a variable decimal point, and hencne can be increased accuracy or more decimal pointns had low signal inntenncities. However, this requires more advanced computing, called the floating point unit or fpu. However, since fixed pointn arithmetic has only a certain amount of decimal points, there is quanntisation errors that exisit. This is where the losses come from.

So the question becomes, how do we address this issue. Well firstly, we cann ensure our quanntisation losses are minimized. To do this, instead of linearly quanntisinng our data, we can follow the distrubiton of radar data, which esseetnnailly follows a Rayleigh distrubtionn. A Rayleigh distrubtionn is data looks like this. To do this, we cann use a piecewise function, which follows the Rayleigh distrubiton at low sigals and follows linenar quanntisationn high signals. This method was tested inn academia on jpeeg and was seem to inncnreasee the peak snnnr from 18 to 21.

The next method is to just use floatinngn point arithmetic when implemntning algorithms. Since floating point is morre complicated, it does increase latency, but since the latencny seen by drhe was so small, at 0.079ms, it is worth invennstigatinng and comparing against.

Next is looking into uses a real life setup where have a full compression and deecompressionn block, such that the setup cann be tested in real life. Here, we cann use a rf-evm module which dsp module underneath for our fpga which plugs into our system on chip, our a pc. Thhee fgpa will do the compression and the pc will then ndo the decompression.

Lastly are the extra compression methods I wish to explore. Firstly, we have the NXP algorithm. As I mentioned before, this algorithm is in nthhe silicon of the miconntroller and so cannot be implemented directly, however it is said to have lossless compression with a compression ratio of 4x, which is better than even the best onne before which was DRHE. Next we have the two image compression methods. These two methods arennt meant directly for real time 3d radar, but would theoretically work just as well, with high compression ratios and low losses. The only concern being latency. Lastly, we hve the most extreme compression method which is adaradar, which has a compression nratio of 100x, with very little losses. The only reason this algorithm is feasible, is the fact that the neural network is to exist onnn the system oin chip rather than at the compression block. However, like the others, the latency is expected to be the main sacrifice.

Now that we have ann idea of what I will be doing in my thesis, we can look at the project plan and timeline. I have set a couple of milestones I wish to hit in my work. Firstly we have the lteraure review and algioritm selection which has been completed. Next, we have the matlab simulation and tradeoff analysis of the various compression methods I wish to implement, inn order to find the best one. Next, we have the setup of the sucesfful implementation nof the comporession algorithm on an fpga and system on nchip. Neext, we have the full setup, with the radar module unit to the compression to the decompression block. Which will then result in the final hardware benchmarking where we will analuse the latency, resource utlisattion nanndn data fidelity of the compression.

Here is a gantt chart which has a little bit more detail regarding thee timelining of my thesis for Thesis B anndn C. Here, we cann see we start with the baselinnen compression model, which connsits of just simulated data, compression and decompression of 1 as a baseline. Then, I wish to look innnto the quanntisation error for drhe to better that algorithm before comparing it against the other compression models. We cannnn then implement on best compression algoriothm onn ann fgpa and decompression onnn a pc before getting the full hardware results.

So what are some of the main challenges of this. Mainly it innvoles the fpga implementation. This means ensuring that minnal resources and latency is introduced on the fpga. This means having a great understanding of fpgas before implementation. Next, we have ensurinngn we get accurate performancnne metrics for each of the meethods, specifically the latency,. This will require investigatinng profiling methods in simulation.

Lastly, we have the skills that I will need to actually accomplish this thesis. My background in work and personal projects has maimnly been in DSP and Embedded Systems. As an Electrical Engineer and Computer Science student, I feel I have the algiorithm design knowledge to accomplish this task. My main connernn though is my FPGA implementnationn. Sinnnce I haven’t donne too much work with them before. However, I am excited to learn and so I believe that it wonnt ennnd up being an issue i nthhe long runn.!