Comparison and Implementation of Data Compression Methods for SoC-based Radar Systems

Student: Jordan Mohindra

Supervisor: A/Prof Beena Ahmed

Assessor: Dr. Mostafa Shahin

Aim of Thesis/Project

- The primary aim of this thesis is to compare and implement various data compression methods for System-on-Chip (SoC) based radar systems.

- We will explore existing gaps in current radar data research by comparing unexamined algorithms.

- This work will be building upon the work of Lukas Kiem, who in his work in 2025, compared various data compression algorithms. Some of my work will be addressing the problems he faced.

General Background

- The rapid expansion of radar sensors in automotive applications (e.g., cruise control, blind spot detection) creates significant challenges in managing large volumes of data.

- The Texas Instruments MIMO radar modules we are using contain 12 transmit and 16 receive antennas, creating a virtual antenna array of 192. Because data scales quadratically with additional antennas, the data throughput increases massively when high resolution is required.

- Current Serializer/Deserializer (SerDes) architectures struggle with this high throughput, requiring expensive shielded cabling and immense computing power at the deserializer while introducing high latency.

- An alternative approach introduces a compression block at each radar module, utilizing an MCU or an FPGA to compress data down to acceptable levels (around 1 Gbps) for Gigabit Ethernet transmission before SoC decompression.

- To evaluate compression techniques, we rely on critical metrics: compression ratio, false positive/negative rates, noise floor (SNR), and total latency.

How We Solve the Problem

- We will first conduct MATLAB simulations to evaluate and compare various commercial and academic compression techniques, including DRHE, AdaRadar, JPEG2000, BAQ, and NXP's algorithm.

- We are using an RF-EVM module where an FPGA handles the compression and plugs into a PC or SoC, which manages the decompression.

- Upon successful implementation, we will benchmark the complete hardware setup to analyze latency, resource utilization on the FPGA, and overall data fidelity.