| Comparison and Implementation of Data Compression Methods for SoC-based Radar Systems Jordan Mohindra · Supervisor: A/Prof Beena Ahmed · School of Electrical Engineering and Telecommunications, UNSW Sydney | [UNSW logo]. |
| --- | --- |

| 1 Motivation Kiem [1]; This thesis. Data; Simulated noise; Real (ColoRadar [2]). Algorithms; DRHE; 8 configurations. Radar; Single TX; 12-TX TDM-MIMO. HW = simulation; No; Yes, byte-exact. Table 1: What this thesis adds 2 Method Raw ADC frames (ColoRadar). ↓. Range FFT (MATLAB). ↓. Predict: DRHE or LPC. ↓. Huffman-code residual. ↓. Compress on FPGA (HLS C++). ↓. Decompress + bit-exact check. Figure 1: Processing chain Parameter; Value. Sensor; TI AWR2243 ×4 cascade. Transmitters; 12 (TDM). Range bins × ramps; 128 × 192. RX channels; 4. Frames tested; 50. Table 2: Test data | 3 Stage 1: screen all Figure 2: Compression vs missed detections (MATLAB, 50 frames) Algorithm; CR; FN %; FP %; Lossless. FP16 (float, reference); 1.00; 0.00; 0.00; –. FX16 (fixed-point baseline); 1.00; 3.77; 1.11; –. DRHE fp16; 1.04; 0.05; 0.01; yes. DRHE; 3.75; 3.77; 1.11; yes. LPC Huffman; 3.70; 3.77; 1.11; yes. BAQ; 3.76; 10.44; 9.69; no. RDVLE; 1.12; 24.01; 4.62; no. RDVLE UDRE; 2.56; 28.73; 4.44; no. FN/FP vs float reference; 3.77 % comes from 16-bit quantisation, not compression Table 3: MATLAB benchmark, 50 ColoRadar frames (highlighted = taken to Stage 2) | 4 Stage 2: DRHE vs LPC Figure 3: CR built up from Huffman coding, prediction and the new dictionary (50 frames) Figure 4: Per-frame CR, re-derived dictionary (MATLAB) |
| --- | --- | --- |
| 5 FPGA system [Insert: Vivado block design, reports/block_design_drhe.pdf]. Figure 5: System on the XCKU5P [Photo: KCU116 board running the test]. Figure 6: Hardware setup | 6 Hardware results Metric; DRHE; LPC; Kiem [1]. CR on board; 3.751; 3.697; 3.17. CR, new dictionary*; 4.218; 4.404; –. Throughput (Gibit/s); 4.77; 0.90; 11.92 †. Encode latency (µs); 13.1; 3,006; 79.4 ‡. Core LUTs; 35,433; 33,704; 45,892. Bit-exact vs sim; 50/50; 50/50; No. * simulation, board run pending † theoretical ‡ incl. FFT Table 4: Measured at 100 MHz on the KCU116 Figure 7: Encode and decode time per frame (decode: board run pending) | 7 Resources Figure 8: Encoder, decoder and both (cores only) 4.40× best lossless CR (LPC*); 0 bits lost, 50/50 frames. 13.1 µs DRHE encode latency; 23 % of LUTs, DRHE enc + dec. Key numbers |

| 8 Conclusion Stage 1: only DRHE and LPC are lossless with a useful CR. Stage 2: LPC compresses most (4.40×), but DRHE is 5× faster with 230× lower latency → DRHE at the sensor. | 9 Next steps Streaming LPC · fixed-point DRHE · stress tests · energy per frame. |
| --- | --- |

[1] L. Kiem, Design and Implementation of Data Compression Methods for SoC-based Radar Systems.   [2] A. Kramer et al., ColoRadar: The direct 3D millimeter wave radar dataset, IJRR, 2022.